AIM: To design an antenna control position system with the mentioned conditions and run the design using Matlab to verify whether the mentioned objectives are reached.

PROCEDURE:

Step 1:  Given Block diagram is first analyzed and θi is assumed to be as angular input,  θ0 as angular output.

Equivalent diagram for the above block diagram is reduced as shown below and Error at the input is made equal to E=1/π(θi-θ0).

Now thus after reducing the above transfer function G(s) is the product of gains of preamplifier power amplifier, motor load and gears.	
Now new G(s) after considering the gain of the output potentiometer is given by

Gs= 20.83ks3+100s2+1.71s2+171s(1π)

Where k is the preamplifier constant

Or else

Gs=  6.6303k/(s3+101.71s2+171s)            (a)

1. Derive the closed loop transfer function

Step 2:

Now consider the closed-loop transfer function for the uncompensated system

We get              G(s)/(1+Gs)      =
              CsRs =   6.630ks3+100s2+1.71s2+171s+6.630k

=     6.630k     s3+101.71s2+171s+6.630k     --> (b)

2. Sketch the root-locus plot of the original system

Step 3:

In order to derive root-locus for the transfer function, consider the original open-loop transfer function           Gs=  6.6303k/(s3+101.71s2+171s).

From the transfer function the numerator and denominator values can be obtained as

num = [0 0 0 6.6303];

den = [1 101.71 171 0];

with varying preamplifier gain k

Matlab code for Root Locus plot of original system

% Root - Locus Plot for the open loop transfer function % G(s) = 6.6303k/(s^3+101.71s^2+171s) num = [0 0 0 6.6303]; den = [1 101.71 171 0]; k = 0:0.4:1000; rlocus(num,den,k); v = [-30 20 -25 25]; axis(v); grid; title('Root Locus for open loop transfer function G(s)= 6.6303k/(s^3+101.71s^2+171s)'); clear;

Root Locus plot generated for above code is :

Fig : 1.1

3. Find a preamplifier gain to achieve 25% maximum overshoot. Describe detail derivations in your project report. Verify your design by the step response using Matlab.

Step 4:    From the root locus in Fig 1.1 the preamplifier gain k is obtained to be approximately 64.6 at  preamplifier  gain of 25.1. Consider equation (b) as below and insert preamplifier k.

CsRs=6.630k     s3+101.71s2+171s+6.630k

So the above transfer function becomes

CsRs=428.31738     s3+101.71s2+171s+428.31738

To verify the design consider step response of the above system

For

num=[0 0 0 428.31738];

den=[1 101.71 171 428.31738];

%unit-step response for closed loop transfer function %428.31738/(s^3+101.71s^2+171s+428.31738) num=[0 0 0 428.31738]; den=[1 101.71 171 428.31738]; t=0:0.1:100; c=step(num,den,t); plot(t,c); grid; clear; title('unit step response of C(s)/R(s)=428.31738/(s^3+101.71s^2+171s+428.31738)')

Step Response for closed loop transfer function C(s)/R(s) is as below

Fig  1.2

From the above step response you can verify the maximum overshoot of 25%.

4. Design a lead-lag compensator using root-locus method, such that closed-loop poles satisfy the fallowing conditions. A) 25% maximum overshoot. B) 2 sec settling time, C) static velocity error constant is 20. Verify your design by ramp and step response using Matlab

- It’s  given maximum   overshoot of 25%, therefore M = 25% (or) = 0.25

M =e-ζ1-(ζ2)π=0.25

Applying natural logarithm on both sides we get

-ζ1-(ζ2)=ln0.25

Or else	                                         ζ2=0.1946-0.1946 (ζ2)

Or else	                                       (ζ2)=0.19461.946

or else                                         (ζ1)=0.403                                                                                         4.1

Given settling time of 2 sec

At (5%) criteria    	t s=3/εωn

Therefore                        ωn=3/ εt s

Or else                             ωn=3.7228

Now in order to design lag lead compensator

Step 1: I considered  γ ≠β  case

For given performance specifications I determined the desired locations for the dominant closed-loop poles.

s2+2sωnζ+ωn2=0

Or else                                                      s2+20.4033.722s+3.7222=0

Or else                                              s2+2.999s+13.853284=0

And closed loop poles are obtained to be

s1= -1.495+i 3.40609

s2  = -1.495-i3.40609

s at s*= -1.495-i3.40609

Step 2: Using the uncompensated open-loop transfer function G(s), angle deficiency ϕm is determined as below if the dominant closed loop poles are at desired location

Fig 4.1

Angle of deficiency   180  -  angleGs|s=s*= ϕm

ϕm= -180-(-113.6897-86.388-1.98037)

Or else                        ϕm=22.05807

Choosing -1.495 as – (1/T1)                                        --------------- (1)

We found  tan180-90-22.05807= 2.46788

Therefore x=(3.406092.46788) = 1.380168404

Therefore extending    ϕm  from complex pole to real axis we found (γ/T1) = (1.380168+1.495) = 2.875168                                                                      -------------------- (2)

So satisfying the requirement angle of (s1 + 1/T1)/(s1 + γ/T1)  T1 and γ  are chosen.

And lead portion is found to be kc (s1 + 1.495)/(s1 + 2.875168404)

Then value of  kc  is determined from magnitude condition

|kc (s1 + 1/T1)/(s1 + γ/T1)  G(s1)| =1

Where Gs1   =  428.33676/(s1   s1   +100s1   +1.71) .

Now substituting the length of the vectors individually we solve with

s+1.71         =0.1252+11.6014=3.4128  ------------------------------- (3)

s+1.495    =3.40609

s+100         =(9703.25+11.60144)

=98.5639                          ----------------------------------------------- (4)

s                         =3.7197411

s+2.8751   =1.382+3.406092

=3.6750304  -------------------------------------------- (5)

From (1) , (2) , (3) , (4) and (5) we obtain

kc   = 3.151924105

From the specified static velocity constant  kv  = 20 sec-1

kv = LtS→0 s . GcsGs

Or else                                                         =       LtS→0 skc (β/γ) G(s)

Solving we get β = 100201.9231(1.71)/(3.1519)(6.6303)(64.6)

Or else                                                   β=4.871

Consider compensated transfer function   with β=4.871    and T2  =10  Where It satisfied Angle and magnitude  condition.

Gc(s) = 3.1519(s+1.495)s+110s+2.8751(s+148.71) Answer for lead-lag compensator

or        = 3.1519s+4.712s+0.1/(s+2.8751)(s+0.0205)

or       =3.1519s2+5.0271s+0.4712/(s2+2.89565s+0.0589022)

Consider original open loop transfer function Gs=6.6303(64.6)/(s2+100s)(s+1.71)

(from (a))

Gc(s)  G(s) Becomes new open loop transfer function of our system

Therefore G(s)Gc(s)   = 428.33676ss+100s+1.71 x 3.1519(s+1.495)s+110s+2.8751(s+148.71)

Solving numerator

= 428.33676 (3.1519)(s+1.495)s+110

Or else                          = 428.31738(3.1519s2+5.02171s+0.04712)

Or else                         = 1350.01355s2+2153.194301s+201.83

Solving Denominator

Or else                           = s+2.8751(s+148.71) ss+100s+1.71

Or else                          = s3+101.71s2+171s(s2+2.895625s+0.059022)

Or else                         =(s5+104.6056s4+465.5720s3+501.15414s2+10.0927s)

Gc(s)  Gs  =

1350.01355s2+2153.194301s+201.83s5+104.6056s4+465.5720s3+501.15414s2+10.0927s

Now closed loop transfer function becomes = C(s)/R(s) =  Gc(s)  G(s)1+Gc(s)  G(s)

Solving numerator                                             = 1350.01355s2+2153.194301s+201.83

Solving denominator                                         =  s5+104.6056s4+465.5720s3+501.15414s2+10.0927s+(1350.01355s2+2153.194301s+201.83)

Or else                                                                  =  s5+104.6056s4+465.5720s3+1851.16769s2+2163.287001s+210.83

Gc(s) G(s)1+Gc(s) G(s) = C(s)/R(s) =(1350.01355s2+2153.194301s+201.83) ( s5+104.6056s4+465.5720s3+1851.16769s2+2163.287001s+210.83)

Verification:

To verify the step and ramp response of the system consider the numerator and denominators of the closed loop transfer functions

num = [1350.01355 2153.194301 201.83];

den = [ 1 104.6056 465.5720 1851.16769 2163.287001 201.83];

Now applying them in step and ramp response Matlab function we obtain

- rise_time              =  0.3800 sec

- peak_time            =  0.9400 sec

- max_overshoot  =   0.2656         = 26.56%

- settling_time       =  1.9200 sec

Matlab Code to find Ramp Response

% Unit Ramp response for closed loop transfer function % C(s)/R(s) = ((1350.01355s^2+2153.194301s+201.83))/( (s^5+104.6056s^4+465.5720s^3+1851.16769s^2+2163.287001s+210.83)) num = [0 1350.01355 2153.194301 201.83]; den = [ 1 104.6056 465.5720 1851.16769 2163.287001 201.83 0]; t = 0:0.1:10; c = step(num,den,t); plot(t,c,'o',t,t,'-') grid title('unit - Ramp response for closed loop transfer function c(s)/R(s)') xlabel('sec') ylabel('input and outputs')

MATLAB code to find Step response , Overshoot and settling time

% C(s)/R(s) = ((1350.01355s^2+2153.194301s+201.83))/( ( s^5+104.6056s^4+465.5720s^3+1851.16769s^2+2163.287001s+210.83))

num = [1350.01355 2153.194301 201.83];

den = [ 1 104.6056 465.5720 1851.16769 2163.287001 201.83];

t = 0:0.02:20;

[y,x,t] = step(num,den,t);

plot(t,y)

grid

title('unit - step response for closed loop transfer function c(s)/R(s)')

xlabel('t(sec)')

ylabel('output y(t)')

r1 = 1; while y(r1) <0.1 , r1 = r1+1;end;

r2=1; while y(r2) <0.9, r2 = r2+1; end;

rise_time = (r2-r1)*0.02

[ymax,tp]=max(y);

peak_time = (tp-1)*0.02

max_overshoot = ymax-1

s = 1001;

while y(s) > 0.95 & y(s) < 1.05; %At (5%) criteria

s=s-1; end;

settling_time = (s-1)*0.02

clear;

% program ref to Modern Control Engineering "K Ogata" - Fourth Edition

Step Response:

Fig 4.2

Ramp Response:

FIG 4.3

- A realization of lag-lead compensator is shown in below figure.

- Derive the overall transfer function of the above circuit.

- Plug in appropriate values for resistors and capacitors, such that your lag-lead compensator in part 4 can be realized by op-amp circuit

- Calculating the transfer functions for lead , lag and middle portion with consant gain individually

We obtain

Gain for the lead portion as = ((R4  ||C2 )|| R5 )

Or else                               = (1/(R5 + (R4||1jωc2))

Since middle portion is an inverting amplifier

Gain for the middle portion = (-R6 /R3)

Gain for the lag portion is     = ((C1+ R2)/(R1 + R2   +C1 ) )

Over all transfer function     =  V0 (t) / Vi (t)

=   (1/(R5 + (R4||1jωc2)) ((C1+ R2)/(R1 + R2   +C1 ) )

(-R6 /R3)

Now to solve for  R1C1R2 consider lead – circuit

Compare the gain for lag portion with answer obtained for lead lag compensator,

Gc(s)  =  3.1519(s+1.495)s+110s+2.8751(s+148.71)             Answer for lead-lag compensator

We obtain

(1/R2   C1 )=0.1     and   (1/(R1 + R2   ) C1) = 0.02502.

Or else on solving the equations on both sides I ended up with

R1 /R2  = 3.87

Now I assumed R1  = 2K,  so I ended up with R2 of 0.5167K.

So  C1 = 0.01935 F

Considering gain for the lead portion

(1/R4   C2 )=2.8751                      (R4 + R5  )/((R4  R5   ) C2)    = 1.495

By solving above equations I obtained (R4    / R5   )  = 1.403

I assumed  R4    = 2K    , then  R5   = 1.425K

And  C2=  0.012 F

Now considering

Constant gain  (1 + R1 R2)  (R  3 /R  6   )  = 0.167

On solving I obtained  R  6   =  R  3 0.034

On assuming  R  3  = 1 k   I obtained  R  6  =  29.4117K

ANS :  C1 = 0.01935 F , C2=  0.012 F, R  3  = 1 k  , R  6  =  29.4117K ,  R4    = 2K

R5   = 1.425K,  R1  = 2K and  R2 of 0.5167K.