AIM:  To regulate the force applied by the pantograph to the catenary to avoid loss of contact due to excessive transition motion. Settling time is reduced along with steady state error made to zero and eliminating high frequency oscillations by cascading a notch filter with the plant.

Given the Feedback block diagram of the system as shown below,

- Derive the closed loop transfer function.

After reducing the transfer function in to transfer function with unity feedback and individual gains of K for controller 1/1000 for actuator and

0.7883(s+53.85)/s2+15.47s+9283s2+8.119s+376.3  for pantograph dynamics and 82300 for spring  along with the notch filter Gn (s) = s2+ 16s+9200/s+602

The final open loop transfer function Gopen(s) = G(s) * Gn (s).

G (s) =  0.6488 s + 34.94(s^4 + 23.59 s^3 + 9785 s^2 + 8.119e004 s + 3.493e006).

Gn(s) = s2+ 16 s + 9200s2+ 120 s + 3600

Gopen(s) =   0.6488 s3+ 45.32 s2+ 6528 s + 3.214e005s6+ 143.6 s5+ 1.622e004 s4+ 1.34e006 s3+ 4.846e007 s2 + 7.115e008 s + 1.258e010

Gclosed (s) = CsRs= Gopen(s) / (Gopen(s) +1)

And K is assumed to be 1 here. Later it is derived while designing PD controller as per the specifications mentioned.

2. Design a PD controller to yield a settling time of approximately 0.3 second with no more than 60% of overshoot.  Plot the step response to verify your design.

Given settling time of 0.3 seconds, we know for 2% criteria ts=4/εωn = 0.3 sec

ζωn=40.3=13.333

And given overshoot of 60% then    M =e-ζ1-(ζ2)π=0.160

Applying  ln (natural log) on both sides we get   ζ2  >  0.160.

And wn= 13.333 / 0.165 = 80.80606061, selecting ζ  as 0.165

subsituting s2+2sωnζ+ωn2=0 we get  s2+26.666s+80.80606061= 0

I obtained complex pole s = -13.333-+i 79.69849774

Now assume a PD controller transfer function of Kp(s+Z) and assume it to be Gc1 (s)

Now to design the PD controller obtain the value of Z which is obtained from the angle condition of

∠GopensGc1 (s)|s=s*= -13.333-+i 79.69849774= -180* ------------------------------------ (1)

By root-Locus method

Zeros and poles of Gopens are obtained to be  -8.000+95.5824i , -8.000+95.5824i  , -53.850  and  
  -7.7356+96.0379i , 7.7356-96.0379i , -60 , -60 , -4.0594+18.9683i , -4.0594-18.9683i.

Now plotting these points on root locus graph to obtain sum of the angles substituted by zeros and poles on the root locus graph with respect to s*we obtain the sum of the angles substituted by zeroes is (63.05205 at 
-53.8500, 251.4411305 at -8.000+95.5824i, 91.90501 at , -8.000+95.5824i which gives  )406.3981905

Similarly by poles 7.7356-96.0379i gives 91.824364711, -7.7356+96.0379i gives 92.8841 , -60 gives 59.64898 , -4.0594-18.9683i gives 98.67939511 results in 656.4129

Now substitute above results in 1 we get θ-250.01480= -180.

Which gives? θ=70.0148.  Now from the  θ  ; Z = 42.31738.

Applying magnitude condition

|(s+z) Kp *K  * Gopens|s=s*  =1 ---------------------------- 2

Where k is the gain of the controller. Which is obtained from root locus plot of open loop transfer function Gopens|

Root Locus plot of Gopens| to find K value

From the above root locus the controller K value is obtained to be 8.07 * 10^3 at an overshoot slightly less than 60.

Equation 2 can be written as kp  s+42.31738.0.6487709s+53.85s2+16s+9200s2+15.47s+9283s2+8.119s+376.3s2+3600+120s=1

At s = -13.333-+i 79.69849774 gives  kp  = (10910.90372/8.07 * 10^3) = 1.352

Now constructing the New transfer function (Gcnew) that includes the PD controller gives the

(GcNew) =

( G (s) =  0.6488 s + 34.94(s^4 + 23.59 s^3 + 9785 s^2 + 8.119e004 s + 3.493e006). *

Gn(s) = s2+ 16 s + 9200s2+ 120 s + 3600 *  Gc1s= 1.352*(s+42.31738) * k controller = 8.07*10^3 )

=

7043 s^4 + 7.9e005 s^3 + 9.169e007 s^2 + 6.488e009 s + 1.477e011

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

Closed loop of (GcNew) is given by = = CsRs= Gc New  / ((GcNew)+1)

= 7043 s^10 + 1.801e006 s^9 + 3.193e008 s^8 + 4.19e010 s^7 + 3.966e012 s^6 + 2.926e014 s^5

+ 1.618e016 s^4 + 5.875e017 s^3 + 1.293e019 s^2 + 1.867e020 s + 1.857e021

-----------------------------------------------------------------------------------------

s^12 + 287.2 s^11 + 6.009e004 s^10 + 9.139e006 s^9 + 1.064e009 s^8 + 1.007e011 s^7

+ 7.564e012 s^6 + 4.492e014 s^5 + 2.085e016 s^4 + 6.902e017 s^3 + 1.465e019 s^2

+ 2.045e020 s + 2.015e021

Step – Response

Matlab-File

% to find step response for PD controlled closed loop transfer function num= [ 7043 1.801e006 3.193e008 4.19e010 3.966e012 2.926e014 1.618e016 5.875e017 1.293e019 1.867e020 1.857e021]; den = [ 1 287.2 6.009e004 9.139e006 1.064e009 1.007e011 7.564e012 4.492e014 2.085e016 6.902e017 1.465e019 2.045e020 2.015e021]; step(num,den); grid

Step Response

From above step response , the +/-2% FINAL STEADY STATE VALUE is within the tolerance band.

As Final steady state value is 0.921 we got  tolerance band is within the 1.02*0.921  = 0.93942

and min value 0.98 *0.921 = 0.90258 we get settling time of approximately …… and overshoot of 55%.

3. Add a PI controller to yield a Zero steady -state error for step inputs. Plot the step response to verify your design.

Now assume a PI controller transfer function of Kp(s+Z)/s and assume it to be Gc2 (s).

Now again consider the root locus plot we used for solving PI parameters , since our new transfer function is Gc2 (s) * Gc1 (s)* Gopens.

Consider the angle condition from 1 again ∠GopensGc1 (s)Gc2 (s) |s=s*= -13.333-+i 79.69849774= -180* ---------> 3

So by solving the above equation we can find the value of Z. To do that we add the angle made by 1/s to the equation 1 which we solved already.

Now we add a value of 99.497234 to the sum of angles made by the poles earlier which is made by (s-0) at origin. These leads to ∠s+z to be -180 -(476.4095428-755.910134) = 279.5005912 – 180 = 99.5005912.

Therefore the value of pole z from equation 3 will be 13.33771664-13.333= - 0.04416643653

Now to find Kp value we need to consider magnitude condition

|(s+z) 1sKp *K * Gopens*Gc1 (s) |s=s*  =1  ------- (4)

We need to modify the equation 2 by  multiplying the magnitude vectors from Gc1 (s)which is |s+42.32| * 1.32 and |s+z| of PI compensator which is |s+0.04416643653| and dividing by |s| which yields us

Kp=10.9950*0.999

Kp=1.0051

That gives  Gc2 (s) = 1.0051*s+0.0441664s

Now  Gc2 (s) * Gc1 (s)* Gopens becomes (Gc2New) =

(1.005*s+0.0441664s*1.352*s+42.31738*0.6488 s + 34.94s4+ 23.59 s3+ 9785 s2+ 8.119e004 s + 3.493e006*8.07 * 103)

That  gives the below equation (Gc2New) open loop

(7115 s^5 + 8.055e005 s^4 + 9.315e007 s^3 + 6.63e009 s^2 + 1.53e011 s + 6.743e009)

(s^7 + 143.6 s^6 + 1.622e004 s^5 + 1.34e006 s^4 + 4.846e007 s^3 + 7.115e008 s^2    + 1.258e010 s)

Step Response:

Consider closed loop transfer function = CsRs= GC2(s) / (Gc2(s) +1) which gives

Step response of the above function from command step(C(s)/R(s)) in matlab command produce the fallowing wave form

Thus we obtained zero steady state error for step inputs.

4. Plot the Bode diagram of the original open-loop transfer function with the notch filter and K =1.

Considering the open loop transfer function Transfer function with Notch filter:

Gopen(s) = G(s) * Gn (s).

G (s) =  0.6488 s + 34.94(s^4 + 23.59 s^3 + 9785 s^2 + 8.119e004 s + 3.493e006).

Gn(s) = s2+ 16 s + 9200s2+ 120 s + 3600 - notch filter transfer function

Gopen(s) =   0.6488 s3+ 45.32 s2+ 6528 s + 3.214e005s6+ 143.6 s5+ 1.622e004 s4+ 1.34e006 s3+ 4.846e007 s2 + 7.115e008 s + 1.258e010

With k = 1

In matlab we obtained fallowing function. Now applying Bode Plot to this original open-loop we obtain.

Transfer function:

0.6488 s^3 + 45.32 s^2 + 6528 s + 3.214e005

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

Bode Plot is obtained as below:

Matlab code for open loop transfer function with k =1

num = [0.6488 45.32 6528 3.214e005];

den = [1 143.6 1.622e004 1.34e006 4.846e007 7.115e008 1.258e010 ];

bode(num,den);

grid

5. Design a lag-lead compensator  using frequency  response method to meet the following specifications: a) at least 35* phase margin b) maximum of 10% of steady state error for a step response. C) at least 35 rad/s bandwidth. Plot the compensated system’s Bode diagram and the step response to verify your design.

Given a steady state error for the step response of 10% or 0.1 that is ess = 0.1

Therefore 1/(1+ kp ) = 0.1  => kp of  value equal to 9.

We know that for lag lead compensator using frequency response method  Gcs= kcs+1T1s+1T2(s+βT1)(s+1βT2)

Step 1: Now to find the value of kc;  kp =Lts→0  GcsGs

That gives  kp = kc*53.85*0.7883*82300*92009283*376.3*105*3600

kc = kp /2.5588 =  3.5214*10^5 .

Step 2: Draw the bode plot of kc*Gs  = G1s
From the bode plot wg is obtained from the phase crossover frequency of G1s  is 30.5.
Therefore zero of the lag portion is obtained to be 1/T2 = 0.1*30.5 = 3.05.   ---- 1

Step3:  Determine the value of β  from phase margin ∅m which is given as 35*. But adding a additional phase margin of 12* gives ∅ to be 47* which gives β value of 10 from formula.
β=(sin∅m-1)/(sin∅m+1)  .   Pole of lag portion is 1/ βT2  = 3.5/10 = 0.305.

Step 4:  To find the parameters of lead portion a 20 db scale of 1 frequancy band (10 to 102) is taken.This is moved on the magnitude plot of  G1s till it meets the –Gm value.

Matlab Code

% bode plot of k_c*G_ (s) = G_1 (s)

num = [ 2.285e005 1.596e007 2.299e009 1.132e011];

den = [1 143.6 1.622e004 1.34e006 4.846e007 7.115e008 1.258e010];

bode(num,den);

grid

That gives us the result of Lead portion.  Where 1/T1 = 11.4 and from the right tip of scale β/T1 = 114.

So now we can construct the lead and lag portion of the compensator as   
Gcs=  3.5214*105((s+11.4)(s+3.05)/(s+114)(s+0.305))

Now to plot the bode plot of compensated system and to verify the bandwidth and phase margin condition consider

Gcs * Gopen(s) = 2.285e005 s5+ 1.926e007 s4+ 2.537e009 s3+ 1.47e011 s2+ 1.715e012 s+ 3.935e012  s8+ 257.9 s7+ 3.266e004 s6+ 3.199e006 s5+ 2.022e008 s4+ 6.297e009 s3+ 9.558e010 s2+ 1.462e012 s + 4.372e011

MATLAB CODE:

% Bode plot of G_c (s) * G_open(s)

num = [2.285e005 1.926e007 2.537e009 1.47e011 1.715e012 3.935e012 ];

den = [1 257.9 3.266e004 3.199e006 2.022e008 6.297e009 9.558e010 1.462e012 4.372e011];

bode(num,den);

grid

From the bode plot thus obtained it is verified whether our system met the required specifications of

- Atleast 35% phase margin b)Maximum of 10% of steady state error.

- Maximum of 10% of steady state error.

%step response for compensated system

num = [2.285e005 7.818e007 1.497e010 2.161e012 2.303e014 1.87e016 1.183e018 5.349e019 1.556e021 2.936e022 4.047e023 2.949e024 6.504e024 1.721e024];

den =[1 515.8 1.318e005 2.347e007 3.199e009 3.408e011 2.904e013 1.988e015 1.069e017 4.438e018 1.412e020 3.354e021 5.709e022 6.898e023 5.17e024 7.783e024 1.912e024 ];

step(num,den);

grid

RESULT :  A PID controller for a given system is designed and required parameters are verified.

A lag-lead compensator for the original system with k =1 is designed and required specifications are verified.

>> H

Transfer function:

s^2 + 16 s + 9200

------------------

s^2 + 120 s + 3600

G = (s*k)/(f*g)

Transfer function:

0.6488 s + 34.94

----------------------------------------------------

s^4 + 23.59 s^3 + 9785 s^2 + 8.119e004 s + 3.493e006

>> G_op = G*H

Transfer function:

0.6488 s^3 + 45.32 s^2 + 6528 s + 3.214e005

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

G_op = G_op = G_op*8.03*10^3

Transfer function:

5210 s^3 + 3.639e005 s^2 + 5.242e007 s + 2.581e009

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

>> d = [1,42.31738];

>> d = tf(d,1);

>> d = 1.352*d

>> GcNew = G_op*Pd

Transfer function:

7043 s^4 + 7.9e005 s^3 + 9.169e007 s^2 + 6.488e009 s + 1.477e011

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

> ClosedGcNew = GcNew/ (GcNew +1)

7043 s^10 + 1.801e006 s^9 + 3.193e008 s^8 + 4.19e010 s^7 + 3.966e012 s^6 + 2.926e014 s^5

+ 1.618e016 s^4 + 5.875e017 s^3 + 1.293e019 s^2 + 1.867e020 s + 1.857e021

-----------------------------------------------------------------------------------------

s^12 + 287.2 s^11 + 6.009e004 s^10 + 9.139e006 s^9 + 1.064e009 s^8 + 1.007e011 s^7

+ 7.564e012 s^6 + 4.492e014 s^5 + 2.085e016 s^4 + 6.902e017 s^3 + 1.465e019 s^2

+ 2.045e020 s + 2.015e021

Tolerance band +/-2%  FINAL STEADY STATE VALUE IS WITHIN THE TOLERANCE BAND ..

1.02*0.921   and min value 0.98 *0.921 ….

>> G_OPEN=T3 / (T4*T5)

Transfer function:

s + 53.85

----------------------------------------------------

s^4 + 23.59 s^3 + 9785 s^2 + 8.119e004 s + 3.493e006

>> G_OPEN= G_OPEN * GnS

Transfer function:

s^3 + 69.85 s^2 + 1.006e004 s + 495420

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

>> G_OPEN = G_OPEN *  0.6488

Transfer function:

0.6488 s^3 + 45.32 s^2 + 6528 s + 3.214e005

----------------------------------------------------------------------------------------

s^6 + 143.6 s^5 + 1.622e004 s^4 + 1.34e006 s^3 + 4.846e007 s^2 + 7.115e008 s + 1.258e010

>> bode(G_OPEN)

>>