Comparison of two algorithms in the automatic segmentation of blood vessels in fundus images

Robert LeAnder, Myneni Sushma Chowdary, and Scott E Umbaugh

ABSTRACT

Effective timing and treatment are critical to saving the sight of patients with diabetes. Lack of screening, as well as a shortage of ophthalmologists, help contribute to approximately 8,000 cases per year of people who lose their sight to diabetic retinopathy, the leading cause of new cases of blindness [1] [2]. Timely treatment for diabetic retinopathy prevents severe vision loss in over 50% of eyes tested [1]. Fundus images can provide information for detecting and monitoring eye-related diseases, like diabetic retinopathy, which if detected early, may help prevent vision loss. Damaged blood vessels can indicate the presence of diabetic retinopathy [9]. So, early detection of damaged vessels in retinal images can provide valuable information about the presence of disease, thereby helping to prevent vision loss. Purpose: The purpose of this study was to compare the effectiveness of two blood vessel segmentation algorithms. Methods: Thirty fundus images from the STARE database were used to develop two algorithms using the CVIPtools software environment. Fifteen original fundus images were used to develop the algorithms. The other fifteen images were derived from the first fifteen and contained opthalmologists’ hand-drawn tracings over the retinal vessels. These expert hand-drawn tracings were used as the “gold standard” for perfect segmentation and compared with the segmented images output by the two algorithms . Both algorithms employ a Laplacian edge detector as the primary means of segmentation. The primary differences between the two algorithms are the steps right before and right after edge detection. Algorithm 1 applies morphological filtering prior to and after edge detection, while Algorithm 2 applies a Yp mean filter before edge detection and an arithmetic mean filter afterward. The segmented images resulting from processing by the two algorithms were visually and quantitatively compared to ophthalmologist hand-drawn images for their effectiveness in segmenting blood vessels out of the original images. Comparisons between the segmented and the hand-drawn images were made using the quantitative measures of Pratt’s Figure of Merit, Signal-to-Noise Ratio and Root Mean Square Error. Visual analysis was made by comparing hand-drawn tracings to the “tracings” done by the algorithms. Results: When the segmented images were compared to the hand-drawn images, it was observed that Algorithm 1 segmented and extracted most of the major vessels and some of the minor ones, but with some intersections missing. Algorithm 2 extracted all the major vessels and fewer minor ones, but left some noise. Algorithm 2 yielded a Pratt’s Figure of Merit of 5.43% more than Algorithm 1. Algorithm 1 beat Algorithm 2 in terms of Signal-to-Noise Ratio and Root Mean Square Error by 14.5% and 1.345%, respectively.

INTRODUCTION

Diabetes causes Diabetic Retinopathy (DR) by damaging the smaller retinal blood vessels which may lead to blindness. DR has three stages: Background Diabetic Retinopathy (BDR), Proliferate Diabetic Retinopathy (PDR) and Severe Diabetic Retinopathy (SDR) [3]. BDR is characterized by arteries that swell, weaken, become damaged and leak blood and serum deposits into the macula (center of the retina). These deposits of protein called exudates make the macula swell and decrease vision. The PDR stage is characterized by problems with retinal circulation and consequent oxygen deprivation. The retinal circulatory system then tries to compensate for circulation loss by re-vascularizing the retinal surface with an abnormal growth of new, fragile vessels to avoid retinal cellular suffocation. However, this process leaks blood into the jelly-filled volume of the eye, thereby increasing pressure and decreasing vision. [3] The purpose of this study is to compare the effectiveness of two blood-vessel-segmentation algorithms. The objective is to choose the best algorithm for refinement and application in the automatic detection of retinal blood vessels damaged in the BDR stage – the earliest stage of DR.

MATERIALS AND METHODS

A. MATERIALS

Image Database: Fifteen color fundus images were collected from the STructured Analysis of the Retina (STARE) image database.

Hand-Drawn Images: Fifteen ophthalmologists’ hand-drawn tracings of the retinal blood vessels in the color fundus images mentioned above were downloaded from the STARE database. These were to be used as the “gold standard” of vessel segmentation and compared to the algorithm-output images to make an assessment of the segmentation effectiveness of those algorithms.

Software: The CVIPtools (Computer Vision and Image Processing) software package was used to perform the image processing operations as well as to calculate the differences between the hand-drawn images and the segmented images output by the two algorithms. Calculation tools in CVIPtools included Pratt’s Figure of Merit, Signal-to-Noise Ratio and Root Mean Square Error.

B. METHODS

Fundus image preprocessing and blood vessel segmentation proceeded as follows (Refer to Figure 1.):

Preprocessing (Algorithms 1 & 2): The images were resized from 150x130 to 300x260 pixels to provide greater visual clarity (See Figures 2 and 3). The green band was extracted from the color fundus images because it contains the greatest amount of contrast, is less affected by variations in illumination and consequently has the most pertinent visual information [8] (See Figures 3 and 4). Both algorithms employ a Laplacian edge detector as the primary segmentation tool. The principal differences between the algorithms happen in preprocessing between green band extraction and edge detection. At that juncture, Algorithm 1 employed a histogram stretch to increase contrast between the blood vessels and the background (fundus) and consequently increased blood vessel details and resolution. (See Figures 4 and 5) [4]. Instead of a histogram stretch, Algorithm 2, employed a Yp mean filter to remove noise and to smooth the images [4] (See Figures 16 and 17). The Yp mean filter was chosen over other filters that were tried because it gave better noise removal and image smoothing. The Yp mean filter is expressed as:

YpMean=(r,c)∈Wd(r,c)pN21p

where d(r,c) are the degraded image pixel values, N is the filter window size and W is the current NxN window centered at d(r,c) [4].

Morphological Filtering (Algorithm 1): In Algorithm 1, after histogram stretching, a morphological filter having a small (size-5) structuring element was used to perform an opening operation. (See Figs. 5 and 6). An opening operation consists of image object erosion followed by dilation. It eliminates all pixels in regions that are too small to contain the structuring element, thereby “smoothing” the vessels’ shapes and enhancing their fundamental geometric properties [4]. “Opening” opens up (expands) holes and erodes edges. Also, due to the ability of the opening operation to remove small noise points, noise patterns were removed. Opening also helped fill in small holes in the vessels while connecting disjoint parts of the vessels that are supposed to be connected [4].

Edge Detection: Both Algorithms 1 and 2 employed a Laplacian edge detector to extract the blood vessels’ features from the image (See Figs. 6-7 and 17-18; also Figs. 1 and 13).

Morphological filtering (Algorithm 1): Next, Algorithm 1 smoothed the vessels through an opening operation using a large (size-15) rectangular structuring element. Using a large-sized structuring element helped extract the finer vessels in the image (see Fig 8). This second morphological filtering step was done to split objects that are connected by narrow strips, and thereby eliminate extraneous peninsulas [4] (see Fig. 8). Afterward, another green-band extraction was done (See Figs. 7-8).

Post Processing: Algorithm 2, at this point, engaged an Arithmetic Mean filter to eliminate noise [4]. The Arithmetic Mean filter is a low pass filter that finds the average of the pixel values in its window and smoothes out local variations within the image [4] (see Fig 19). An attempt to reconstruct missing vessel intersections resulting from segmentation was made using an edge-linking technique (See Figure 22). Edge linking links edge points to create segments and boundaries [4].

Both algorithms converted the images from color, to gray scale, to binary images. Then, a logical NOT operation was performed ( see Figs 20 -21 and 9 -10).

At this point, because Algorithm 1 had extracted most of the major and minor vessels with some missing intersections and bifurcations, a Hough transform was used to reintegrate vessel segments [4] (see Fig 11). The Hough algorithm takes a collection of edges points (found by the Laplacian edge detector) and finds all the lines on which these edge points lie [4].

Evaluation Tools: The algorithms were evaluated using the following evaluation tools:

- Pratt’s Figure of Merit measures the success of an edge detector by comparing the distances between edges in an original image to the edges in its edge-detected counterpart. It ranges from 0 – 1. The Pratt’s Figure of Merit for a missing edge is 0 (0%) and for a perfectly detected edge is 1 (100%).

- Signal to Noise Ratio: It is an objective measure used to measure the amount of error. The output image is compared with the hand drawn image to measure the signal strength [4].

- Root Mean Square (RMS) Error: This is also an objective criteria used to determine the amount of error.

Original Image

Resize

Green Band Extraction

Histogram Stretch

Morphological Operation-Opening with size 5

rectangular structuring element

Laplacian Edge Detection

Morphological Operation-Opening with size 15

rectangular structuring element

Color to Gray Conversion

Binary Thresholding

Not Operation

Hough Transform

Final Image

Fig. 1: Flowchart of algorithm 1 for automatic segmentation of bloodvessels in fundus images using CVIP tools.

Fig 2: Original Image

Fig 3: Resized Original Image (300x260)

Fig 4: Preprocessing Step 1. Extraction of the green band from the resized original color image helps enhance image details.

Fig 5: Preprocessing Step 2.  Histogramic stretch of the green band image helps increase detail enhancement

Fig 6: Morphological Operation: Opening operation with a size-5 rectangular structure produces smoothing effect and enhances the details with respect to their spatial properties

.

Fig 7: Edge Detection:

Edges are detected from the morphologically operated image using laplacian edge detector

Fig 8: Morphological operation

opening operation with size 15 rectangular structure element splits objects that are connected by narrow strips and eliminates peninsulas from edge detected image

Fig 9: Post processing step 1

Color to gray scale conversion: This is the intermediate step to convert the color image to binary format

Fig 10: Post processing step 2

Grayscale to binary conversion: Gray scale image is converted to binary by thresholding

Fig 11: Post processing step 3

Not operation & Hough Transform

Not operation produces the black background and the missing intersections were tried to integrate using Hough transform

Fig 12: Binary Converted Hand drawn image from stare database

Original Image

Resizing

Green Band Extraction

Pre process Yp mean Filtering to remove noise

Laplacian edge detection

Post Processing Arithmetic mean Filtering to remove noise

Color to Gray Conversion

Binary Thresholding

Not operation

Edge Linking

Final Image

Fig. 13: Flowchart of algorithm 2 for automatic segmentation of blood vessels in fundus images using CVIP tools.

Fig. 14: Original Image

Fig. 15: Resized Original Image (300x260)

Fig 16: Preprocessing step 1

Extraction of the green band from the resized original color image helps enhance image details.

Fig 17: Preprocessing step 2

Yp mean filtering eliminates noise from the green band extracted image there by producing a smoothed effect

Fig 18: Edge detection

The features of the image are extracted by the laplacian edge detector

Fig 19: Post processing step 1

Arithmetic mean filter is applied on the edge detected image to remove noise

Fig 20: Post processing step 2

Color to gray scale conversion is the intermediate step to convert the color image to binary image

Fig 21: Post processing step 3

Grayscale to binary conversion: Gray scale image is converted to binary by thresholding

Fig 22: Not operation & Edge linking

Not operation gives black background and the missing intersections were tried to reintegrate using edge linking

Fig 23: Binary converted Hand drawn image from stare database. It is thresholded at a value 75

RESULTS : (THIS IS THE GENERAL DESCRIPTION OF THE RESULTS THAT ARE COMMON FOR BOTH THE ALGORITHMS)

The images were analyzed to compare the algorithms’ extraction effectiveness, using ophthalmologists’ hand drawn images given in Stare database. The hand drawn images were converted to binary form in order to make comparison. The hand drawn images are subjected to colorgray scalebinary conversion. The gray scale image is thresholded at a value of 75. Comparison parameters included Signal to Noise Ratio (SNR), Root Mean Square (RMS) error and Pratt’s Figure of Merit.  (See tables 1, 2, 3 and figures 37, 38, 39 in the results section.). Pratt’s Figure of merit, Signal to Noise Ratio and Root Mean Square error are the objective fidelity criteria that are used for measuring the amount of error in a reconstructed image by comparing it with a known image [4]. Objective fidelity criteria are not always correlated with our perception of the image quality. For example, an image which has low error as determined by RMS error value may look worse than an image with high error value. These measures are useful for relative comparison of different versions of same image [4].

I. COMPARISON OF THE RESULTS OF ALGORITHMS WITH THE HAND-DRAWN IMAGES

Image A

Fig 25: Original Image A (from the STARE database

Fig 26: Binary format of ophthalmologist’s hand-drawn tracing of blood vessels in Original Image A (from STARE database)

Fig 27: Final Image Showing the Result of Segmenting Blood Vessels Using Algorithm 1

Degree of match to the Hand-drawn image using

Pratt’s figure of Merit:  0.6506

Signal to Noise ratio:     12.14

Root mean square error: 63.027

Fig 28: Final Image Showing the Result of Segmenting Blood Vessels Using Algorithm 2

Degree of match to the Hand-drawn Image using

Pratt’s figure of Merit:  0.6685

Signal to Noise ratio:     10.536

Root mean square error: 65.810

IMAGE B

Fig 29: Original Image B from stare database

Fig 34: Binary format of ophthalmologist’s hand-drawn tracing of blood vessels in Original Image B (from STARE database)

Fig 31: Final Image Showing the Result of Segmenting Blood Vessels Using Algorithm 1

Degree of match to the Hand-drawn image using

Pratt’s figure of Merit:  0.5361

Signal to Noise ratio:     11.11

Root mean square error: 70.967

Fig 32: Final Image Showing the Result of Segmenting Blood Vessels Using Algorithm 2

Degree of match to the Hand-drawn image using

Pratt’s figure of Merit:  0.5577

Signal to Noise ratio:     10.136

Root mean square error: 69.389

IMAGE C

Fig 33: Original Image, C from stare database

Fig 34: Binary converted Hand drawn image from stare database

Fig 35: Final Image Showing the Result of Segmenting Blood Vessels Using Algorithm 1

Degree of match to the Hand-drawn image using

Pratt’s figure of Merit:  0.6418

Signal to Noise ratio:     11.669

Root mean square error: 66.545

Fig 36: Final Image Showing the Result of Segmenting Blood Vessels Using Algorithm 2

Degree of match to the Hand-drawn image using

Pratt’s figure of Merit:  0.5822

Signal to Noise ratio:     10.859

Root mean square error: 63.044

- Pratt’s Figure of Merit:  It is an objective measure which ranges from 0 – 1. The Pratt’s Figure of Merit for a missing edge is 0 (0%) and for a perfectly detected edge is 1 (100%). If there are no missing valid edge points, classifying noise pulses as valid edge points and no smearing of edges, the detected edge is said to be ideal edge. This metric assigns better rating to smeared edges than to missing edges. This is because, there are techniques to determine smeared edges but it is difficult to determine missing edges [4].

Fig 37. Bar graph for results of experiment . The Bar Set representing the Pratt’s Figure of Merit (FOM) for 15 Test images from STARE database.  The data table below the graph shows the approximated values of the FOM for Algorithm 1 and Algorithm 2. The FOM values > 0.5 are approximated to 1 and with FOM values < 0.5 are approximated to 0. The average Pratt’s Figure of merit for 15 test images using Algorithm 1 is 48.84% and for algorithm 2 is 54.27%.

| Images | Pratt’s FOM for Algorithm 1 | Pratt’s FOM for Algorithm 2 |
| --- | --- | --- |
| Image 1 | 0.6506 | 0.6685 |
| Image 2 | 0.5361 | 0.5577 |
| Image 3 | 0.6418 | 0.5825 |
| Image 4 | 0.4877 | 0.5164 |
| Image 5 | 0.5972 | 0.5429 |
| Image 6 | 0.6197 | 0.5734 |
| Image 7 | 0.4996 | 0.5800 |
| Image 8 | 0.5102 | 0.5610 |
| Image 9 | 0.3820 | 0.4453 |
| Image 10 | 0.3421 | 0.4513 |
| Image 11 | 0.4885 | 0.4961 |
| Image 12 | 0.4414 | 0.5158 |
| Image 13 | 0.3592 | 0.5245 |
| Image 14 | 0.3503 | 0.5930 |
| Image 15 | 0.4205 | 0.5328 |

Table 1. Results of Pratt’s Figure of Merit(FOM) for Algorithm 1 and Algorithm 2 on 15 fundus images from Stare database. The average Pratt’s Figure of merit for 15 images using algorithm 1 is 48.84% and for algorithm 2 is 54.27%

- Signal to Noise Ratio: It is an objective measure that is used to measure the amount of error. The processed image is compared with the hand drawn image to measure the signal strength [4]. This parameter gives the strength of the blood vessels extracted.

Fig 38. Bar graph for Signal to noise ratio results of experiment . The bar Set represents the Signal to noise ratio (SNR) for 15 images from Stare Database. The data table below the graph shows the approximated values of the SNR for Algorithm 1 and Algorithm 2. The images with SNR are approximated to their nearer values as shown in data table. The average SNR for the images obtained from Algorithm 1 is 11.404 and that from Algorithm 2 is 9.959

| Images | SNR for Algorithm 1 | SNR for Algorithm 2 |
| --- | --- | --- |
| Image 1 | 12.14 | 10.536 |
| Image 2 | 11.11 | 10.136 |
| Image 3 | 11.669 | 10.859 |
| Image 4 | 10.774 | 9.859 |
| Image 5 | 12.952 | 9.055 |
| Image 6 | 11.915 | 9.749 |
| Image 7 | 12.296 | 10.419 |
| Image 8 | 11.961 | 9.981 |
| Image 9 | 10.595 | 9.736 |
| Image 10 | 10.948 | 9.950 |
| Image 11 | 10.166 | 9.016 |
| Image 12 | 10.698 | 9.744 |
| Image 13 | 11.747 | 10.124 |
| Image 14 | 11.30 | 10.873 |
| Image 15 | 10.794 | 9.356 |

Table 2. Results of Signal to noise ratio(SNR) for Algorithm 1 and Algorithm 2 on 15 fundus images from Stare database. The average SNR for the images obtained from Algorithm 1 is 11.404 and that from Algorithm 2 is 9.959

- RMS Error: The difference between the standard pixel value (original) and modified (reconstructed) pixel value is considered as error. It is not desirable if the positive and  negative errors to get cancelled. So, individual pixel error is squared [4]. The square root of the error squared divided by the total number of pixels in the image gives the root mean square error. This is an objective criteria that determines the amount of error .

Fig 39. Bar graph for Root Mean Square (RMS) Error results of experiment: The bar Set represents the Root mean square error (RMS) for 15 images from Stare Database. The data table below the graph shows the approximated values of the RMS error for Algorithm 1 and Algorithm 2. The images with RMS values are approximated to their nearer values as shown in data table. The average RMS error obtained for the images obtained from Algorithm 1 is 69.13 and from Algorithm 2 are 70.06

| Images | RMS error for Algorithm 1 | RMS error for Algorithm 2 |
| --- | --- | --- |
| Image 1 | 63.027 | 65.810 |
| Image 2 | 70.967 | 69.389 |
| Image 3 | 66.545 | 63.044 |
| Image 4 | 73.760 | 71.773 |
| Image 5 | 57.407 | 70.435 |
| Image 6 | 64.684 | 73.000 |
| Image 7 | 61.910 | 66.837 |
| Image 8 | 64.339 | 70.814 |
| Image 9 | 75.295 | 73.122 |
| Image 10 | 72.303 | 71.105 |
| Image 11 | 79.108 | 80.307 |
| Image 12 | 79.730 | 73.048 |
| Image 13 | 65.994 | 69.492 |
| Image 14 | 69.429 | 62.924 |
| Image 15 | 73.595 | 69.823 |

Table 3. Results of Root Mean Square error (RMS) for Algorithm 1 and Algorithm 2 on 15 fundus images from Stare database. The average RMS error obtained for the images obtained from Algorithm 1 is 69.13 and from Algorithm 2 are 70.06

II. DISCUSSION

The algorithms developed for automatic segmentation of blood vessels in fundus images using CVIPtools are experimented on 15 images from stare database and the final results are compared with the hand drawn images from the stare database. Algorithm 1 segmented the image by filling out holes and smoothing out object outlines.  However, some of the intersections are missing.  These missing intersections were tried to reintegrate using Hough transform. Even though the Hough transform is performed, not all the missing vessels were integrated (see Fig 11). Algorithm 2 extracted the blood vessels by histogram modification and edge detection followed by mean filtering to remove the noise. The obtained results are analyzed in terms of SNR (signal to noise ratio), RMS (root mean square) error and Pratt’s figure of merit. For this metric FOM will be 1 for a perfect edge. This metric assigns a better rating to smeared edges than to offset or missing edges. In this method the ideal edge image i.e. the hand drawn image is compared with edge detection image i.e., the final result and the scaling factor (1/9) is used to adjust the penalty of offset edges. Since some of the vessels are missing, error occurs when the final images are compared with binary converted hand drawn images. This error affects the signal strength. The outer ring is not eliminated, which may contribute to the noise and this could be the reason for high values of RMS error in both the algorithms. The final results obtained from the algorithms are binary images, whereas the hand drawn images are color images, so the hand drawn images are converted to binary format (Color --> Grayscale --> Binary) at a binary threshold value of 75. During the course of experiments, it was observed that better results could be achieved in terms of SNR, RMS error and Pratt's FOM if the outer ring is eliminated.

ANALYSIS

From the above results, we can say that on an average, we are able to extract above 56% of the required result using algorithms 1 and 2.

- Algorithm 1 worked better for few images when compared to others.

- Algorithm 2 gave us the constant results except that there is noise existing in the image.

- The average Pratt’s Figure Of merit for the images obtained  from Algorithm 1 is 55% and from Algorithm 2 is 50%, the results from Pratt’s FOM  shows that algorithm1 worked well for  most images when compared to others and algorithm 2 gave constant results for most of the images. (see table 1 in results section)

- The average SNR for the images obtained from Algorithm 1 is 9.14 and that from Algorithm 2 is 8.60. (see table 2 in results section)

- The average RMS error obtained for the images obtained from Algorithm 1 is 69 and from Algorithm 2 are 70. As the minor vessels and intersections are missing in the images the RMS error may be high. (see table 3 in results section)

CONCLUSION:

In this Project we proposed two algorithms for automatic detection and Segmentation of blood vessels in Fundus Images using CVIP tools. Fifteen images have been experimented for both the algorithms. The major difference between the algorithms was that Algorithm 1 had difficulty with intersections and bifurcations. To overcome this, we tried to perform a reconstruction process using “Hough transform”, this could recover the intersections of major vessels, but with all the detected minor vessels missing and, moreover, the SNR and RMS error values for these images were not desirable. Algorithm 1 extracted most of the major vessels, while Algorithm 2 extracted all of the major blood vessels and many of the minor ones. Current work includes refinement of minor vessel extraction.

Significance of This Present Work:

Algorithm 2's effectiveness may lead to:

- Better detection and characterization of stages of disease by general practitioners of medicine.

- Earlier diagnosis and treatment.

- Improved efficiency in screening process, resulting in efficient mass screenings of patients.

- Better monitoring of disease stages and improvement resulting from treatment.

# IV. References (2 used references and proper formatting)

[1] Teng, T., Lefley, M., Claremont, D., Progress towards automatic diabetic ocular screening: a review of image analysis and intelligent systems for diabetic retinopathy, Medical and Biological Engineering and Computing, Vol. 40(1): 2-13, 2002

[2] Meadows, M., Saving your sight: Early detection is critical, FDA Consumer Magazine, March-April 2002. http://www.fda.gov/fdac/features/2002/202_eyes.html

[3] Iqbal, M., Aibinu, A., Gubbal, N., Automatic Diagnosis of Diabetic Retinopathy Using Fundus Images (masters thesis), Bleking Institute of Technology, Karlskrona Sweden, 2006

[4] S.E. Umbaugh, “Computer Imaging: Digital Image Analysis and Processing”; CRC Press; January 2005.

[5] Reconstruction of Vascular Structure in Retinal Images.  - Bin Fang, Wynne Hsu, Mong Li Lee, Department of Computer Science, school of computing, National University of Singapore.

[6] Segmentation of Vessel-Like Patterns Using Mathematical Morphology and curvature evaluation. – Frederic Zana and Jean-Claude klein

[7] An improved matched filter for blood vessel detection of digital retinal images. -  Mohammed Al-   Rawi∗, Munib Qutaishat, Mohammed Arrar - Computer Science Department, King Abdullah the Second School for Information Technology, University of Jordan, P.O. Box 13496, Amman 11942, Jordan. Received 29 August 2005; received in revised form 27 February 2006; accepted 16 March 2006

[8] Detection and segmentation of drusen deposits on human retina: Potential in the diagnosis of age-related macular degeneration - K. Rapantzikos, M. Zervakis*, K. Balas

Department of Electronic Computer Engineering, Digital Image and Signal Processing Laboratory, Technical University of Crete, GR-73100 Crete, GreeceReceived 27 July 2001; received in revised form 1 January 2002; accepted 21 May 2002

16

[9] http://www.southstaffordshirepct.nhs.uk/services/diabetes/docs/retinopathy.pdf

SHAPE